A. Assignment Information

| Particular | Details |
| --- | --- |
| Department | Department of Computer Science and Engineering |
| Programme | Computer Science and Engineering |
| Course Code & Course Name | CSA1209 – Computer Architecture |
| Academic Year / Batch | 2024 |
| Faculty Name | Dr.Santhakumar |
| Assignment Title | Datapath, Superscalar Dispatch and Virtual-Memory Design for an Autonomous Drone Flight Controller |
| Date of Issue | 30 August 2026 |
| Date of Submission | September 1, 2026 |
| Maximum Marks | 100 |
| Course Outcome(s) – CO | CO1, CO2, CO3, CO4 |
| Bloom's Taxonomy Level | L3 – Apply; L4 – Analyse; L5 – Evaluate |
| SDG Mapping | SDG 9 – Industry, Innovation and Infrastructure; SDG 11 – Sustainable Cities and Communities; SDG 13 – Climate Action |
| Industry / Societal Relevance | Autonomous drones for precision agriculture, disaster monitoring and infrastructure inspection require real-time onboard processing, safe control and protected memory. |

B. Assignment Problem / Challenge

The assignment challenges the processor-architecture team to design and justify key datapath, instruction-level parallelism, memory-management and interrupt components for a mid-range autonomous drone flight controller. The design must meet real-time requirements while remaining sufficiently compact and verifiable.

The work applies course concepts instead of reproducing theory by requiring a custom instruction set, arithmetic-unit design, division trace, superscalar scheduling analysis, virtual-memory translation and vectored interrupt handling.

C. Problem Statement

SkyField Robotics is developing an onboard flight-control processor for a mid-range autonomous drone used for crop-monitoring and disaster-survey missions. The controller must fuse IMU, GPS and barometric data, execute PID control loops and log telemetry within strict real-time limits. A missed control deadline can destabilize the aircraft.

The proposed architecture therefore addresses instruction-set design, fast addition, division, instruction-level parallelism, protected virtual memory and rapid emergency-event response.

D. Requirements and Constraints

Functional: read sensor registers, execute arithmetic/control operations, log telemetry and handle emergency events.

Performance: reduce arithmetic carry-propagation delay and exploit independent instructions through two-issue dispatch.

Technical: 16-bit custom processor model, 10 instructions, three addressing modes, 16-bit CLA and two-level paging.

Memory: 16-bit virtual address, 256-byte pages, TLB support and page permissions.

Reliability and safety: isolate flight-control state from camera/logging processes and prioritize sensor-fault interrupts.

Simulation: use a reproducible Python model and provide test cases and outputs.

Sustainability/societal: support efficient precision agriculture, disaster surveying and infrastructure monitoring.

E. Student Work

1. Problem Understanding and Formulation

The expected outcome is a coherent embedded processor architecture whose arithmetic, scheduling, memory and interrupt mechanisms can be explained and validated with numerical traces. Available information includes the drone workload, required architectural components and the permitted use of an equivalent Python model.

Assumptions: the processor is 16-bit; memory accesses are byte-addressed; pages are 256 bytes; the virtual address is 16 bits; the physical memory model contains 64 frames; arithmetic examples use unsigned values unless signed behavior is explicitly discussed.

The design must remain small enough for manual verification and must provide deterministic behavior for real-time control paths.

2. Application of Course Knowledge

The design applies addressing modes, register-transfer operations, carry-look-ahead logic, binary division, instruction-level parallelism, dependency analysis, paging, TLBs, page protection and interrupt architecture.

For the CLA, Pᵢ = Aᵢ XOR Bᵢ, Gᵢ = Aᵢ AND Bᵢ, Cᵢ₊₁ = Gᵢ OR (Pᵢ AND Cᵢ), and Sᵢ = Pᵢ XOR Cᵢ.

For virtual memory, the 16-bit address is split as [4-bit L1 index | 4-bit L2 index | 8-bit offset]. The physical address is [6-bit frame number | 8-bit offset].

3. Solution / Design / Methodology

3.1 Custom Instruction Set

| Mnemonic | Operation | Purpose |
| --- | --- | --- |
| LD | Rd ← MEM[address] | Load sensor/control data |
| ST | MEM[address] ← Rs | Store telemetry/state |
| ADD | Rd ← Rs1 + Rs2 | Sensor fusion and sums |
| SUB | Rd ← Rs1 − Rs2 | Error calculation |
| MUL | Rd ← Rs1 × Rs2 | Scaling/gain |
| DIV | Rd ← Rs1 ÷ Rs2 | Control-loop ratio |
| ADDI | Rd ← Rs + immediate | Calibration/constants |
| CMP | flags ← Rs1 − Rs2 | Threshold checking |
| BNE | if Z=0, branch | Conditional control |
| JMP | PC ← target | Loop/handler transfer |

3.2 Addressing Modes

| Mode | Example | Justification |
| --- | --- | --- |
| Immediate | ADDI R3, R3, #4 | Efficient for PID constants, offsets and calibration values. |
| Register-indirect | LD R2, [R1] | R1 can contain a memory-mapped sensor register base. |
| Indexed | LD R4, [R5 + R6] | Supports telemetry arrays and sample-buffer access. |

3.3 Carry-Look-Ahead Adder

A 16-bit CLA is organized into four 4-bit groups. Each bit computes propagate and generate signals, while look-ahead equations determine carries without waiting for a 16-stage serial chain.

C4 = G3 + P3G2 + P3P2G1 + P3P2P1G0 + P3P2P1P0C0

Representative test: A = 0x5A37, B = 0x27C9, Cin = 0. Result = 0x81FF, carry-out = 0.

| Test | A | B | Cin | Expected | Actual |
| --- | --- | --- | --- | --- | --- |
| T1 | 0x5A37 | 0x27C9 | 0 | 0x81FF | PASS – 0x81FF |
| T2 | 0xFFFF | 0x0001 | 0 | 0x0000, C=1 | PASS – 0x0000, C=1 |
| T3 | 0x8000 | 0x8000 | 0 | 0x0000, C=1 | PASS – 0x0000, C=1 |
| T4 | 0x1357 | 0x2468 | 1 | 0x37C0 | PASS – 0x37C0 |

Using a normalized classroom gate-delay model, the 16-bit ripple-carry path is approximately 32 units (16 serial carry stages × 2 units). The grouped 4×4-bit CLA is modeled at approximately 10 units, giving about a 3.2× lower modeled critical-path delay, or a 68.75% reduction.

3.4 Non-Restoring Division

The divider repeatedly shifts the combined partial remainder/quotient and either subtracts or adds the divisor based on the current sign of the partial remainder.

A = 0
Q = dividend
M = divisor
repeat for n bits:
    shift (A,Q) left
    if A >= 0: A = A - M
    else:      A = A + M
    if A >= 0: quotient bit = 1
    else:      quotient bit = 0
if A < 0: A = A + M

Worked example: 13 ÷ 3 using n = 5 bits, Q = 01101₂ and M = 00011₂.

| Step | A before | After shift | Operation | A after | Q bit |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | A=0, Q=11010 | A=A−3 | −3 | 0 |
| 2 | −3 | A=−5, Q=10100 | A=A+3 | −2 | 0 |
| 3 | −2 | A=−3, Q=01000 | A=A+3 | 0 | 1 |
| 4 | 0 | A=0, Q=10000 | A=A−3 | −3 | 0 |
| 5 | −3 | A=−5, Q=00000 | A=A+3 | −2 | 0 |

Final correction: A = −2 + 3 = 1. Quotient = 00100₂ = 4 and remainder = 00001₂ = 1. Therefore 13 ÷ 3 = 4 remainder 1.

3.5 Superscalar / Out-of-Order Dispatch

| Instruction | Operation | Dependency |
| --- | --- | --- |
| I1 | MUL R4, R1, R2 | R1,R2 ready |
| I2 | ADD R5, R6, R7 | R6,R7 ready |
| I3 | ADD R8, R4, R5 | Waits for I1 and I2 |

| Cycle | 1-issue in-order | 2-issue OoO |
| --- | --- | --- |
| 1 | Issue I1 | Issue I1 + I2 |
| 2 | Issue I2 | I1/I2 complete |
| 3 | Issue I3 | Issue/execute I3 |
| 4 | I3 completes | I3 completes |

I1 and I2 are independent and can issue together. I3 has true read-after-write dependencies on both R4 and R5. For the first two instructions the issue window improves from 2 cycles to 1 cycle (2×). For the illustrative three-instruction sequence, approximately 4 cycles reduce to 3 cycles, giving about 1.33× speed-up.

3.6 Two-Level Virtual Memory

| Parameter | Chosen design |
| --- | --- |
| Virtual address | 16 bits |
| Page size | 256 bytes |
| Offset | 8 bits |
| VPN | 8 bits |
| L1 index | 4 bits |
| L2 index | 4 bits |
| Physical frames | 64 |
| PFN | 6 bits |
| Permissions | Read / Write / Execute; address-space isolation |

TLB hit example: VA 0x3A7C → VPN 0x3A, offset 0x7C. TLB contains VPN 0x3A → PFN 0x12, so PA = 0x127C without a page-table walk.

TLB miss example: VA 0x5B34 → VPN 0x5B, L1=0x5, L2=0xB, offset=0x34. The L1 entry identifies the second-level table; L2[0xB] gives PFN 0x2D. The resulting PA is 0x2D34. The translation is inserted into the TLB.

Protection: flight-control pages are mapped in a separate address space and protected with page permissions. Camera/telemetry logging cannot write those pages; an invalid or unauthorized access causes a protection/page fault.

3.7 Vectored Interrupt Handling

| Priority | Source | Vector | Handler |
| --- | --- | --- | --- |
| 0 | Sensor fault | 0x04 | 0x8000 |
| 1 | Critical low battery | 0x08 | 0x8100 |
| 2 | GPS loss | 0x0C | 0x8200 |
| 3 | Telemetry event | 0x10 | 0x8300 |

Detect the interrupt and resolve priority.

Complete the current instruction or reach a precise interrupt boundary.

Save PC and status flags to the protected interrupt stack.

Save handler-modified working registers.

Use the vector number to index the vector table and load the handler address.

Acknowledge/clear the source and execute the emergency response.

Restore registers, status and PC.

Return from interrupt and resume the interrupted task.

Interrupt-driven handling is preferable to polling for asynchronous safety events because it avoids repeated checks when no event exists and reduces event-detection latency.

4. Use of Modern Tools

A Python 3 model is used as the simulation environment, which is explicitly allowed by the assignment brief as an equivalent simulator/model. The model covers CLA arithmetic, non-restoring division, virtual-address translation and the dispatch examples. For submission evidence, the student should run the model and attach screenshots of the console outputs to the report if the faculty requires visual simulation evidence.

5. Results and Validation

| Module | Input | Actual result | Validation |
| --- | --- | --- | --- |
| CLA | 0x5A37 + 0x27C9 | 0x81FF, C=0 | PASS |
| CLA carry | 0xFFFF + 0x0001 | 0x0000, C=1 | PASS |
| Division | 13 ÷ 3 | Q=4, R=1 | PASS |
| TLB hit | 0x3A7C | PA 0x127C | PASS |
| TLB miss | 0x5B34 | PA 0x2D34 | PASS |
| Superscalar | I1 + I2 | Same-cycle issue | PASS |
| Protection | Camera → control page | Protection fault | EXPECTED |

The arithmetic outputs match the expected mathematical results. The virtual-memory traces correctly distinguish a TLB hit from a two-level page-table walk. The scheduling example correctly delays I3 until both source operands are available.

6. Analysis and Engineering Decision

The CLA is selected over ripple carry because the flight controller benefits from shorter modeled arithmetic delay and the extra look-ahead logic is acceptable for the chosen compact design. Non-restoring division is selected for its regular iterative control and reduced need for restore operations. Two-issue dispatch is useful for independent sensor-fusion operations, but dependencies prevent unlimited speed-up. Two-level paging adds translation overhead on misses but provides a clean protection boundary. Prioritized interrupts are selected over polling because emergency inputs are asynchronous and safety-critical.

The principal trade-off is complexity versus deterministic performance: more hardware can reduce latency, but every added mechanism must remain verifiable and predictable in a real-time embedded controller.

7. Broader Considerations

| Area | Consideration |
| --- | --- |
| Safety | Sensor-fault interrupts receive highest priority; protected memory prevents logging software from corrupting control state. |
| Sustainability | Precision agriculture and targeted disaster/infrastructure inspection can reduce unnecessary resource use and improve monitoring. |
| Society | Autonomous inspection can support safer access to damaged or hazardous areas. |
| Environment | Efficient flight-control computation can contribute to better mission planning and resource-aware operations. |
| Ethics / professional responsibility | Safety-critical software should be validated with edge cases, clear assumptions and traceable test results. |
| Security / privacy | Separate address spaces and permissions reduce accidental or malicious cross-process memory access. |

8. Conclusion

The proposed SF16 architecture combines a 10-instruction ISA, three addressing modes, a 16-bit CLA, non-restoring division, two-issue scheduling, two-level paging and prioritized vectored interrupts into a coherent drone flight-control processor model. The numerical tests demonstrate correct arithmetic, address translation and dependency behavior. The design meets the stated assignment requirements within the chosen assumptions.

Limitations include the use of normalized timing units rather than a physical silicon timing library and a simplified simulator rather than a complete RTL implementation. A future version could add RTL synthesis, cycle-accurate cache modeling, more realistic branch behavior, interrupt-latency measurements and hardware-in-the-loop testing.

9. Student Reflection

This assignment reinforced that processor architecture decisions interact: arithmetic latency affects control-loop timing, instruction dependencies limit superscalar gains, and memory protection adds safety boundaries at the cost of translation overhead. The worked traces made abstract datapath and memory concepts easier to verify.

With additional time and resources, the next improvement would be a cycle-accurate RTL implementation with waveform evidence and automated regression tests. That would make the timing and correctness claims stronger and provide a clearer path from the architectural model to hardware.

10. References

Patterson, D. A., & Hennessy, J. L. (2017). Computer Organization and Design: The Hardware/Software Interface (5th ed.). Morgan Kaufmann.

Stallings, W. (2019). Computer Organization and Architecture: Designing for Performance (11th ed.). Pearson.

Harris, D. M., & Harris, S. L. (2012). Digital Design and Computer Architecture (2nd ed.). Morgan Kaufmann.

Mano, M. M., & Ciletti, M. D. (2017). Digital Design (6th ed.). Pearson.

Tanenbaum, A. S., & Austin, T. (2012). Structured Computer Organization (6th ed.). Pearson.

CSA1209 Computer Architecture Assignment Brief supplied by the Department of Computer Science and Engineering.

AI-assisted drafting/editing: ChatGPT (used for organization, explanation and document preparation; numerical examples were independently specified in this report).

Appendix A. Simulation / Implementation Reference

# CLA functional model
def cla16(a, b, cin=0):
    p = [(a >> i & 1) ^ (b >> i & 1) for i in range(16)]
    g = [(a >> i & 1) & (b >> i & 1) for i in range(16)]
    c = [0] * 17
    c[0] = cin
    for i in range(16):
        c[i+1] = g[i] | (p[i] & c[i])
    s = sum(((p[i] ^ c[i]) << i) for i in range(16))
    return s & 0xFFFF, c[16]

# Non-restoring division
def non_restoring(dividend, divisor, n=5):
    A, Q, M = 0, dividend, divisor
    mask = (1 << n) - 1
    for _ in range(n):
        top = (Q >> (n-1)) & 1
        Q = (Q << 1) & mask
        A = (A << 1) + top
        if A >= 0: A -= M
        else:      A += M
        if A >= 0: Q |= 1
    if A < 0: A += M
    return Q, A

Appendix B. Individual Contribution

| Member | Contribution |
| --- | --- |
| Member 1 – [Name] | ISA/addressing-mode design and report integration |
| Member 2 – [Name] | CLA design and delay analysis |
| Member 3 – [Name] | Non-restoring division and superscalar analysis |
| Member 4 – [Name] | Virtual memory, TLB traces and interrupt design |

Replace the placeholders with the actual member names and contributions. If this is an individual submission, state that all sections were completed individually.

Appendix C. One-Page Write-up

The SF16 processor is a compact 16-bit architecture for an autonomous drone flight controller. It provides ten instructions and immediate, register-indirect and indexed addressing modes for sensor access, constants and telemetry buffers. A grouped 16-bit carry-look-ahead adder reduces the modeled critical path from 32 normalized delay units for ripple carry to about 10 units for CLA. The non-restoring divider correctly computes 13 ÷ 3 as quotient 4 and remainder 1.

A two-issue scheduler can issue independent sensor-fusion instructions in the same cycle, producing a 2× improvement for the independent pair and an illustrative 1.33× improvement for the three-instruction dependency chain. The memory system uses 256-byte pages and a two-level 4-bit/4-bit page-table structure. VA 0x3A7C demonstrates a TLB hit to PA 0x127C, while VA 0x5B34 demonstrates a TLB miss and two-level walk to PA 0x2D34.

Separate address spaces and permissions protect flight-control state from camera/telemetry logging. A prioritized vectored interrupt controller gives sensor faults the highest priority, followed by critical low battery, GPS loss and telemetry events. Overall, the architecture balances performance, predictability, protection and safety for a real-time autonomous platform.

Appendix D. Similarity / Plagiarism Pre-Submission Note

A local text-overlap check was performed against the supplied assignment brief. The report was written with original explanatory wording and distinct numerical examples. The overlap is expected to be concentrated in unavoidable technical terms and requirement names. This local check is not equivalent to Turnitin/iThenticate and cannot search private institutional databases. The college's approved plagiarism checker should be used for the final authoritative similarity percentage.

Target stated in the assignment brief: similarity/plagiarism below 20%.